P053 Tester Muti-DUTx4

2019/12/23  v1.2

By CNHsu

| Date | Version | Description |
| --- | --- | --- |
| 2019/9/16 | V0.1 | First draft |
| 2019/9/19 | V0.2 | Modify reg. |
| 2019/9/20 | V0.3 | +step motor |
| 2019/9/24 | V0.4 | +BT addr |
| 2019/9/26 | V0.5 | Revise some text +uart log reference |
| 2019/10/1 | V0.6 | +freq. test log |
| 2019/10/2 | V0.7 | Replace PWM.h to tone library +’q’ to stop pwm output Can direct drive speaker for “talk mic “ test |
| 2019/10/18 | V0.8 | +servo *remove step moto function |
| 2019/10/19 | V0.9 | +auto key press command (single/tir) |
| 2019/11/05 | V1.0 | + more description |
| 2019/12/16 | V1.1 | +modified some typo |
| 2019/12/23 | V1.2 | + pcba option |

System block

UART=38400

UART=57600

PC

UART=115200

PCBA

Mega328p

(arduino nano)

- 5V/4V EN

- Check V/I

- Check Tx

- PWM &FFT

- LED

12V

5V

4V

5V, 4V,TX,MIC,SP

Atmega2560

- Select pcba

- Command & results

- BT control

- Motor control?

PCBA

Mega328p

(arduino nano)

- 5V/4V EN

- Check V/I

- Check Tx

- PWM &FFT

- LED

12V

5V

4V

5V, 4V,TX,MIC,SP

PCBA

Mega328p

(arduino nano)

- 5V/4V EN

- Check V/I

- Check Tx

- PWM &FFT

- LED

12V

5V

4V

5V, 4V,TX,MIC,SP

PCBA

Mega328p

(arduino nano)

- 5V/4V EN

- Check V/I

- Check Tx

- PWM &FFT

- LED

12V

5V

4V

5V, 4V,TX,MIC,SP

Tx,Rx

Setup environment:

- BES FW: 38400

\\192.168.1.50\p053\RDDOC\治具\開發文件\test-multi\bes-fw

- Arduino FW: \\192.168.1.50\p053\RDDOC\治具\開發文件\test-multi\arduino

- sch: \\192.168.1.50\p053\RDDOC\治具\開發文件\test-multi\sch

- uart speed : BES to nano(38400), nano to mega(57600), mega to PC (115200)

- mega uart port:

serial0(to pc): usb to pc

serial1(to BT):tx(18)rx(19)

serial2(to pcba1):tx(16)rx(17) …..  bes rx only

serial3(to pcba2):tx(14)rx(15) …..  bes rx only

serial4(to pcba3):tx(48)rx(50) …..  bes rx only

serial5(to pcba4):tx(49)rx(51) …..  bes rx only

- mega driver: \\192.168.1.50\p053\RDDOC\治具\開發文件\test-multi\arduino

Atmega2560 to mega328p (char): to nano board

Example: 1 means char ‘1’, command to nano board

V/I control & test

| Command (char) | Action | Description |
| --- | --- | --- |
| 1 | 5V on | Uart: “(1)**5 ON\n\r” (?) depends pcba |
| 2 | 5V off | “(1)**5 OFF\n\r” |
| 3 | 4V on | “(1)**4V ON\n\r” |
| 4 | 4V off | “(1)**4 OFF\n\r” |
| 5 | Check 5V voltage | “(1)**5 voltage\n” “5VV”????? ? show voltage value |
| 6 | Check 5V current | “(1)**5 current\n” “5VI”???? ? show current value |
| 7 | Check 4V voltage | “(1)**5 voltage\n” “4VV”????? ? show voltage value |
| 8 | Check 4V current | “(1)**5 current\n” “4VI”???? ? show current value |
| 9 | Set uA | “(1)**5V uA\n\r” |
| a | Set mA | “(1)**5V mA\n\r” |
| B | Set uA | “(1)**4V uA\n\r” |
| c | Set mA | “(1)**4V mA\n\r” |
| d | PWM=500Hz PWM2=1KHz | “(1)**PWM” D9 D9=1k,D3=500 \n\r” |
| e | PWM=1KHz PWM2=500Hz | “(1)**PWM” D3 D9=500,D3=1k \n\r” |
| f | LED7=1 |  |
| g | LED7=0 |  |
| h | LED8=1 |  |
| I | LED8=0 |  |
| n | Reset nano board | “**Reset nano 0.5sec D44,45,46,47 toggle Low-delay 0.5s- high *nano ‘low’ reset |
| q | Stop tone | Stop pin9,3 pwm output |
| s | Check USB5V status ‘1’=OK ‘0’=fail | “(1)**FLT state\n\r” |
| x | Read R/L frequency | “(1)**FFT \n\r” L=A7, R=A5 Show: Data: FFTL=???? Data: FFTR=???? ? show frequency |
| z | Check key log And BT address | “(1)**wait...” “press key now .....” Will count down 5,4,3,2,1… “# Powe on “#PWR” “#V-“ “#v+” “#ANC” “#PWR off” ** trip-click at normal power on state, can get BT address “#BT : “ address string ** time out ~7sec |

PCBA DUT and signals select(Atmega2560 only):

| Command prefix “#” (char), ex. ‘#1” | Action | Description |
| --- | --- | --- |
| #1 Or #2 or #3 or #4 | Select pcba 1 or 2, or 3 or 4 | “select serial2\n\r” “select serial3\n\r” “select serial4\n\r” “select serial5\n\r” *default pcba=1 |

*for multi-port test

Should select pcba ‘#?’ firstly before other operation. (default pcba select ‘1’)

HC-05 PIO 2-8 & PIO 11(Atmega2560 only):

// “PIO=2,1” // port-2 hight,

// “PIO=7,0” // port-7 low

| Command prefix “p” (char), ex. ‘pio2,1” | Action | Description |
| --- | --- | --- |
| PIO=2,1 | BT IO 2=high | Uart :”**HC-05 PIO\n\r” BT uart :”AT”+PIO2,1 BT uart:”AT+MPIO\n\r “ Return PIO status |

Bluetooth command (atmega2560 only):

| Command (char), ex. ‘1’ | Action | Description |
| --- | --- | --- |
| u | “AT+RESET\n\r “ Delay(1000) “AT+INIT\n\r “ Delay(1000) “AT+state\n\r “ delay(500); “AT+INQM=1,1,3\n\r delay(500); “AT+class=240404\n\r Delay(500) | “**LED1-BT INIT\n\r” Initial BT ** include 3.5sec delay |
| v | Check BT RSSI | “**LED1-BT rx: scan\n\r” “BT scan\n\r “ Rssi=???? ? BT energy ? ? loop counter 0-9 “---end scan ---\n\r” **if scan fail, repeat ‘v’ command to re-scan |
| w | Check INQM status | “**BT INQM\n\r” It should be 1,1,3 |

BEStest_BT_5 and later

Disable step motor function

Step motor and other (atmega2560 only):

| Command prefix “%” Command (char), ex. “%s100” | Action | Description |
| --- | --- | --- |
| n | Reset nano board | D44,45,46,47 toggle Low-delay 0.5s- high *nano ‘low’ reset |
| s | Speed ‘rmp’ | “set stepper speed(rpm) : “ |
| d | steps | “set stepper steps : “ - full cycle=4096 - neg-value counterclockwise *after stpes, set all INTx to low |
| r | running | Run with pre-set value in ‘s’ &’d’ |
| t | Terminal step motor | Set all INTx to low |

*step motor: 28BYJ-48 (5V, 4096steps for 1-cycle)

Servo motor and other (atmega2560 only):

Servo CH1:D22,23,24,25 (servo0,1,2,3)

Servo CH2:D26,27,28,29(servo4,5,6,7)

Servo CH3:D30,31,32,33(servo8,9,10,11)

Servo CH4:D34,35,36,37(servo12,13,14,15)

| Command prefix “%” Command (char), ex. “%i11” | Action | Description |
| --- | --- | --- |
| s[] | Set servo ‘sit’ position deg | "reset servo 'sit' ????" %s100: set all servo ‘sit’ to 100deg (range 1~175) |
| u[] | Set servo ‘up’ position deg | "reset servo 'up' ???? %u90: set all servo ‘up’ to 90deg (range 1~175) |
| d[] | Set servo ‘down’ position deg | "reset servo 'down' ??? %d45: set all servo ‘down’ to 45deg (range 1~175) |
| i[x][y][z] | set specific servo’s sit or up or down deg | “sit/up/down to servo [y] to [z]” Set servo z, x=s/u/d, z=deg %is01100: set servo1 ‘sit’ deg100 %iu1245: set servo12 ‘up deg45 Valid range: x: ‘s’,’d’,’u’ y:0-15 z: 1-175 (deg) |
| r[x] | Run servo[x] With pre-define Sit,up,down | “servo x run” Sit Up Down Sit Example: %r5 Servo 5 auto run “sit-up -down-sit” |
| k[x] | Auto key press | “"press key:??” %k[0]-[3] single press %k[4]-[7] tri-press Pcba channel should pre-defined with “#” command Ex. %k3: pcbach key-3 single press %k6: pcbach key-2 tri-press |
| z[x] | Auto key press And Check pcba uart log | %z[0]-[3] single press %z[4]-[7] tri-press Pcba channel should pre-defined with “#” command Ex. %z3: pcbach key-3 single press %z6: pcbach key-2 tri-press *host can check pcba response uart log |

Command ‘d’, ‘e’ and ‘x’ test log

MICP

Sp+, sp-

MICP: Arduino freq. output

SP+SP-: BES speaker output

Check FFT

Check FFT

Set 500Hz

Set 1KHz

To pcba

Sp+, sp-

tester board option

To 2560

servo

Option to pcba or audio-jack

Pwm to bes-pcba

Bes-pcba to fft

整型波~650mV

Debug only

BES-PCBA UART 3.3 or 5V

BES-PCBA 5V source

Pwm to spk 5V方波

To PCBA

UART 5V or 3.3V

5v from reg. or usb

To host